Tuto install OS raspberry Pi 3B

Telecharger RaspBerry Pi Imager v1.8.5

Modele de Raspbery Pi : Raspberry Pi 3

OS : RaspBerry Pi OS (64 bit)

Stockage : MASS STORAGE DEVICE USB

Trouver l’IP du raspberry :

- Ifconfig

……

wlan0 :……

Inet 192.168.123

….

ssh pierre@192.168.1.23

password : topgun12

en cas de réinstallation si ce message s’affiche :

@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@

@    WARNING: REMOTE HOST IDENTIFICATION HAS CHANGED!     @

@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@@

IT IS POSSIBLE THAT SOMEONE IS DOING SOMETHING NASTY!

etc

ssh-keygen -R 192.168.1.23

cablage jack composite video :

pi 3 - What is the 3.5mm port standard in RPi3? - Raspberry Pi Stack Exchange

- Sleeve - Composite video (PP24, according to Raspberry Pi 3 test pads, though not listed in the official schematics.)

- Ring 2 - Ground (PP6, among others)

- Ring 1 - Audio-Right (also PP26)

- Tip - Audio-Left (also PP25)

//////////////////////////////////////////////////////////////////////////////////////////////////////////

Cablage ecran LED :

Pin

Vcc	  	2

Gnd	 	6

CS 	 	29

Reset

DC	 	3

SDI(MOSI) 	19

SCK		23

LED(BL)	1

Enable spi : sudo raspi-config  / interface options / enable spi

Installation driver ILI9488 :

### Update

sudo apt update

sudo apt upgrade

sudo reboot

### Headers

sudo apt install git bc bison flex libssl-dev libncurses5-dev

sudo apt-get install raspberrypi-kernel-headers

### ti9488

git clone https://github.com/Vasily-Kapustin/ti9488

cd ti9488

make

sudo cp ili9488.ko /lib/modules/`uname -r`/kernel/drivers/gpu/drm/tiny/

sudo depmod

If you cannot find the /build directory this means that the headers did not download correctly

### Set up overlay

cd rpi-overlays

sudo dtc -@ -I dts -O dtb -o /boot/overlays/generictft-9488-overlay.dtbo generictft-9488-overlay.dts

### /boot/firmware/config.txt

dtoverlay=generictft-9488-overlay

dtparam=speed=62000000

dtparam=rotation=90

référence : Vasily-Kapustin/ti9488

//////////////////////////////////////////////////////////////////////////////////////////////////////////

Vcc blanc

Gnd noir

CS orange

Reset marron

D/C rouge

SDI(MOSI) jaune

SCK bleu

BL violet

SDO vert

///////////////IR REMOTE///////////////////////////////////////

Dans boot/firmware/config.txt ajouter (sudo nano config.txt) :

dtoverlay=gpio-ir,gpio_pin=26

sda connecté sur GPIO 26

sudo reboot

sudo apt install ir-keytable

sudo pip3 install evdev --break-system-packages

sudo apt-get install evtest

sudo ir-keytable -t -p all -s rc0

sudo evtest (choisir gpio-ir_recv)

3

08         09         10        11

16         17         18        19

24         25         26

32                                   35

40         41        42          43

48         49        50          51

56         57        58

03     06    09

02     05    08

01     04    07

4.92 cm

| Câble 1 | Gpio AW | pin RSPi | GPIO RSPi |
| --- | --- | --- | --- |
| Orange | 31 | 35 | 19 |
| Violet | 33 | 37 | 26 |
| Jaune | 35 | 36 | 16 |
| noir | 37 | 38 | 20 |
| vert | 39 | 40 | 21 |

| Câble 2 | Gpio AW | pin RSPi | GPIO RSPi |
| --- | --- | --- | --- |
| Bleu | 30 | 3 | 2 |
| jaune | 32 | 5 | 3 |
| rouge | 34 | 7 | 4 |
| blanc | 36 | 15 | 22 |
| noir | 38 | 12 | 18 |
| Orange | Data_IR | 13 | 27 |
| Marron | Vcc_IR 5V | 2 |  |
| violet | 21 Gnd | 6 |  |
| vert | 53 Vcc 3.3V | 1 |  |

| GPIO LCD |  | pin RSPi | GPIO RSPi |
| --- | --- | --- | --- |
| Vdd | blanc | 17 |  |
| Gnd | noir | 25 |  |
| CS | orange | 24 | 8 |
| Rst | marron | 22 | 25 |
| D/C | rouge | 18 | 24 |
| SDL | jaune | 19 | 10 |
| SCK | bleu | 23 | 11 |
| BL | violet | 17 |  |
| SDO | vert | 21 | 9 |

Modifier le circuit pour faire fonctionner le poussoir du codeur rotatif (GPIO 22) : relier une broche du codeur à la masse :

Installation TKinter (vérifier que le module n’est pas déjà installé par défaut) :

sudo apt-get install python3-tk

installer la lib ttk

sudo apt-get install python3-pil python3-pil.imagetk

supprimer le process python running :

kill -9 $(ps |grep python | awk '{print $1}')